PAWS: A WebAssembly Stack Coprocessor with Hardware-Isolated Sandboxing for Secure & Efficient Scientific IronClaw AI Agent Tools

Qiuping Wu1*, Mugeng Liu1, Yixuan Zhang1, Hongxiao Zhao1, Yihan Fu1

1Peking University, Beijing, China

*Corresponding Email: wuqiuping@stu.pku.edu.cn

- Motivation

Current large language models (LLMs) struggle to perform accurate and reliable scientific computing analysis due to hallucinations. Therefore, for scientific computing, AI agents such as OpenClaw can be employed to analyze user requirements, invoke specialized scientific computing tools, and subsequently analyze the results—without relying on the LLM to perform the computations themselves.

Among the various AI agents, IronClaw stands out due to its remarkable advantages in security and extensibility. This is because the tools invoked by IronClaw are implemented as WebAssembly (Wasm) plugins. Wasm is a binary format supporting multiple programming languages, offering high computational efficiency along with a sandbox security mechanism. However, mainstream processors adopt architectures based on register files (e.g. x86,  ARM), which are not compatible with the stack-based structure of Wasm bytecodes, resulting in runtime overhead, including program startup latency, increased memory footprint, and unpredictable execution behavior.

To enable IronClaw to efficiently and securely invoke local computation-intensive scientific computing tools built on Wasm, we propose PAWS, a native Wasm coprocessor that directly executes Wasm bytecode in hardware to eliminate runtime overhead and provide a secure hardware sandbox. It is highly suitable for IronClaw's scientific-oriented compute-intensive applications.

- Technical Novelty

We use digital circuits to realize:

- Native Wasm Execution: we present the first microprocessor (to the best of our knowledge) capable of executing complete Wasm bytecode without the need of JIT or interpreter.

- Hardware Stacks: we replace conventional register files with a dual-stack design, eliminating stack emulation overhead and reducing memory access latency.

- WASM Block-Based Control Flow: We introduce a control stack mechanism that directly supports Wasm’s structured control flow, avoiding costly software translation.

- Deterministic and Low-Overhead Execution: PAWS provides predictable execution latency and near-zero startup overhead, making it suitable for real-time and edge scenarios.

- Implementation & Results

- The PAWS soft core is deployed on the FPGA AX7325b (xc7k325tffg900-1), operating at a frequency of 50 MHz, with an on-chip power consumption of 0.58 W.

- Benchmark: ① PolyBench benchmark suite. ② Benchmarks inspired by IronClaw. ③ Phase-wise time breakdown benchmarks.

- PAWS achieves 5.99–34.10× speedup vs. Intel Xeon platforms w/ software Wasm runtime.

- PAWS achieves 16.78–107.57× speedup vs. NVIDIA Jetson TX2 w/ software Wasm runtime.

- PAWS achieves 44.42× speedup vs. Raspberry Pico W.

- These improvements stem from the elimination of runtime overhead and the reduction of memory access latency.

Fig. 1 Overall architecture and system of PAWS and its potential to accelerate IronClaw tools.

Fig. 2 Multiple evaluations for different scenarios.

Fig. 3 PAWS test systems and demo videos.

System demonstration video of PAWS: https://vimeo.com/1137154052

Web API demonstration video of PAWS: https://vimeo.com/1177946987

ClawBench inspired by IronClaw: https://anonymous.4open.science/r/BF65/

IronClaw is OpenClaw inspired implementation implemented by Rust with Wasm on privacy and security.